| # MATERIAL SAFETY DATA SHEET |  |
| --- | --- |
| 1. Chemical Identification: |  |
| PRODUCT NAME: | ANISEED OIL |
| BOTANICAL NAME: | PIMPINELLA ANISUM |
| INCI NAME: | PIMPINELLA ANISUM (ANISE)SEED OIL |
| CAS #: | 8007 – 70 -3 |
| COUNTY OF ORIGIN: | INDIA |
| 2. Classification: |  |
| Chemical Identification: ESSENTIAL OIL, ANISEED OIL UN No. 2319: Terpene hydrocarbons, n.o.s. Packaging Group: III Class: 3 |  |
| 3. Hazardous Ingredients Information: |  |
| MIXTURE OF SEVERAL NATURAL ISOLATES |  |
| 4. First Aid Measures |  |
| Eye Contact: Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Get medical attention. Skin Contact: In case of contact, immediately flush skin with plenty of water. Cover the irritated skin with an emollient. Remove contaminated clothing and shoes. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get Medical attention. Serious Skin Contact: Wash with a disinfectant soap and cover the contaminated skin with an anti-bacterial cream. Seek medical attention. Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. Serious Inhalation: Not available. Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband. Serious Ingestion: Not available. |  |
| 5. Fire/Explosion Hazard: |  |
| Flammability of the Product: May be combustible at high temperature. Auto-Ignition Temperature: Not available. Flash Points: <110 °C Flammable Limits: Not available. Products of Combustion: Not available. Fire Hazards in Presence of Various Substances: Slightly flammable to flammable in presence of open flames and sparks, of heat. Non-flammable in presence of shocks. Explosion Hazards in Presence of Various Substances: Risks of explosion of the product in presence of mechanical impact: Not available. Risks of explosion of the product in presence of static discharge: Not available. Fire Fighting Media and Instructions: SMALL FIRE: Use DRY chemical powder. LARGE FIRE: Use water spray, fog or foam. Do not use water jet. Special Remarks on Fire Hazards: Not available. Special Remarks on Explosion Hazards: Not available. |  |
| 6. Accidental Release Measures |  |
| Small Spill: Absorb with an inert material and put the spilled material in an appropriate waste disposal. Large Spill: Absorb with an inert material and put the spilled material in an appropriate waste disposal. Finish cleaning by spreading water on the contaminated surface and allow evacuating through the sanitary system. |  |
| 7. Handling and Storage |  |
| Precautions: Keep away from heat. Keep away from sources of ignition. Empty containers pose a fire risk; evaporate the residue under a fume hood. Ground all equipment containing material. Do not breathe gas/fumes/ vapor/spray. Wear suitable protective clothing. In case of insufficient ventilation, wear suitable respiratory equipment. If you feel unwell, seek medical attention and show the label when possible. Avoid contact with skin and eyes. Keep away from incompatibles such as oxidizing agents. Storage: Keep container tightly closed. Keep container in a cool, well-ventilated area. Do not store above 25°C (77°F)... |  |
| 8. Exposure Controls/Personal Protection |  |
| Engineering Controls: Provide exhaust ventilation or other engineering controls to keep the airborne concentrations of vapors below their respective threshold limit value. Ensure that eyewash stations and safety showers are proximal to the work-station location. Personal Protection: Splash goggles. Lab coat. Vapor respirator. Be sure to use an approved/certified respirator or equivalent. Gloves. Personal Protection in Case of a Large Spill: Splash goggles. Full suit. Vapor respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product. Exposure Limits: Not available. |  |
| 9. Physical & Chemical Properties: |  |
| APPEARANCE:; COLOURLESS TO PALE YELLOW CLEAR LIQUID. ODOR:; STRONG CHARATERISTIC ODOR OF ANISE LIQCORICE. SOLUBILITY:; SOLUBLE IN ALCOHOL AND OILS. INSOLUBLE IN WATER.. SPECIFIC GRAVITY:; 0.975 – 0.992 @ 20°C. OPTICAL ROTATION:; -. REFRACTIVE INDEX:; 1.552 – 1.560 @ 20°C. FLASH POINT; 95°C. EXTRACTION METHOD:; STEAM DISTILLATION OF ROOTS. ACTIVE CONSTITUENT; ANETHOLE – 92%+. |
| 10. Stability and Reactivity Data |  |
| Stability: The product is stable. Instability Temperature: Not available. Conditions of Instability: Excess heat (elevated temperatures), flame, ignition sources Incompatibility with various substances: Reactive with oxidizing agents. Corrosivity: Non-corrosive in presence of glass. Special Remarks on Reactivity: Not available. Special Remarks on Corrosivity: Not available. Polymerization: Will not occur. |  |
| 11. Toxicological Information |  |
| Routes of Entry: Absorbed through skin. Eye contact. Toxicity to Animals: LD50: Not available. LC50: Not available. Chronic Effects on Humans: Not available. Other Toxic Effects on Humans: Hazardous in case of skin contact (irritant), of ingestion, of inhalation. Special Remarks on Toxicity to Animals: Not available. Special Remarks on Chronic Effects on Humans: Not available. Special Remarks on other Toxic Effects on Humans: N/A Acute Potential Health Effects: N/A Skin: May cause skin irritation. N/A Eyes: May cause eye irritation. N/A Hazards for inhalation and ingestion have not been established and the toxicological properties of this substance have not been fully investigated. Acceptable industrial hygiene practices should be maintained. Limit exposure to product whenever possible. Wash after contact. Thoroughly wash any contaminated clothing. Respiratory protection may not be not normally be required if substance is used in a well ventilated area. However, the use of NIOSH approved respiratory protection may be required is substanced is used in a confined, poorly ventilated area. |  |
| 12. Ecological Information |  |
| Ecotoxicity: Not available. BOD5 and COD: Not available. Products of Biodegradation: Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise. Toxicity of the Products of Biodegradation: The products of degradation are more toxic. Special Remarks on the Products of Biodegradation: Not available. |  |
| 13. Disposal Considerations |  |
| Waste Disposal: Waste must be disposed of in accordance with federal, state and local environmental control regulations. |  |
| 14. Transport Information |  |
| DOT Classification: Not a DOT controlled material Identification: Not applicable. Special Provisions for Transport: Not applicable. |  |
| 15. Regulatory Information |  |
| Federal and State Regulations: No products were found. Other Regulations: Not available. Other Classifications: Vapor respirator. Be sure to use an approved/certified respirator or equivalent. Splash goggles. |  |
| Disclaimer: The information contained in this Certificate of Analysis and Material Safety Data Sheet is obtained from current and reliable sources. As the ordinary or otherwise use(s) of this product is outside the control of Phytos Natur CZ s.r.o.., no representation or warranty, expressed or implied is made as to the effect(s) of such use(s) (including damage or injury), or the results obtained. Phytos Natur CZ s.r.o.. expressly disclaims responsibility as to the ordinary or otherwise use(s). Furthermore, Phytos Natur CZ s.r.o. as to the fitness for any use should consider nothing container herein as a recommendation. The liability of Phytos Natur CZ s.r.o.. is limited to the value of the goods and does not include any consequential loss. |  |